RF Remote Control for Electrical Control Panel

The project consists of two parts i.e. Control Panel and Remote controller. The Remote controller design is based on ATMEL AT89C2051 Microcontroller. The AT89C2051 (20pin DIP IC) makes the remote controller design as simple & compact. The remote controller is powered by 9V Battery.

Block Diagram of Remote Controller

The remote control has a small keypad, which is used to control the equipments. The key pressed by the user is read by the microcontroller. The key value is send to the DTMF (Dual Tone Multi-Frequency) encoder. The encoder generates voice tone for  that particular key value. The Tone is modulated (AM) with Carrier Frequency (433MHZ) and transmitted using RF module.

The control panel design is based on ATMEL AT89C51 Microcontroller. It is designed in such way to control 4 different equipment / Appliance using Relays. Also this Panel produces one variable Power source, which can be used to vary the speed of any Motor / Fan.

Error: Reference source not found

Block Diagram of Control Panel

The RF signal transmitted by remote control is received by RF receiver module. The output of the RF Receiver will be DTMF signal. This DTMF signal is decoded and the binary value is generated using the decoder. The AT89C51 microcontroller gets the signal from the DTMF decoder and based on the information the microcontroller changes the output status of the relays. The controller does not drive the relays. So the relay driver circuitry is implemented in between the microcontroller and relays. The microcontroller can also vary the power applied to the motor / Fan by varying the firing angle of a TRIAC. The information about varying the motor power is read from the remote controller.